WROOM #2

Bluetooth Hands-Free Profile Integration

Voice-Driven Obstacle Alert System — ESP-IDF Implementation Guide

Platform: ESP32 / ESP32-WROOM-32  •  Framework: ESP-IDF (Bluedroid Classic)

Companion module: WROOM #1 (navigation / obstacle-sensing unit)

# Overview

This guide sets up WROOM #2 as a Bluetooth Hands-Free Profile (HFP) Audio Gateway using ESP-IDF, based on Espressif's official hfp_ag example. In this design, the Bluetooth headset acts like the HFP client — the same role a headset plays with a mobile phone — while WROOM #2 acts as the Audio Gateway itself. That relationship is what makes two-way audio possible: microphone audio flows in from the headset, and spoken alerts flow back out to its speaker.

WROOM #2 sits between the person, the Bluetooth headset, and WROOM #1 (the navigation/obstacle-sensing module). It turns a spoken or button-triggered request into a UART command, waits for WROOM #1's answer, and converts that answer into a short spoken response played back through the headset.

## System Flow

Bluetooth headset microphone

↓

HFP / SCO audio → WROOM #2 receives raw voice samples

↓

Speech-command recognition

↓

Command: IS_THERE_AN_OBSTACLE

↓

UART → WROOM #1

↓

Object, direction & distance result

↓

UART → WROOM #2

↓

Prerecorded PCM response

↓

HFP / SCO audio → Bluetooth headset speaker

# 1. Install ESP-IDF

Install the current stable release of ESP-IDF using Espressif's installer or the VS Code extension. Then copy the official example into its own project rather than editing it in place:

cd %IDF_PATH%\examples\bluetooth\bluedroid\classic_bt\hfp_ag

Create a separate project copy rather than modifying the original example, then target the original ESP32:

idf.py set-target esp32 idf.py menuconfig

NOTE Use the original ESP32 / WROOM-32 target, because Bluetooth Classic is required. The Bluedroid stack is what provides Bluetooth Classic profile support — the ESP32-S3/C3 Bluetooth LE-only variants will not work here.

# 2. Enable Bluetooth HFP Audio

In idf.py menuconfig, enable Bluetooth and Bluedroid Classic Bluetooth. Menu names vary slightly across ESP-IDF releases, but look under:

Component config └─ Bluetooth ├─ Bluetooth ├─ Bluedroid ├─ Classic Bluetooth └─ HFP Audio Gateway

For headset microphone and speaker audio delivered through application callbacks, use the Voice over HCI / SCO audio path supported by the HFP configuration.

# 3. Initialise the HFP Audio Gateway

The startup sequence should follow this order — Bluetooth controller and Bluedroid stack first, then GAP, then HFP:

nvs_flash_init(); esp_bt_controller_init(&bt_cfg); esp_bt_controller_enable(ESP_BT_MODE_CLASSIC_BT); esp_bluedroid_init(); esp_bluedroid_enable(); esp_bt_gap_register_callback(bt_gap_callback); esp_hf_ag_register_callback(hfp_event_callback); esp_hf_ag_init(remote_headset_address);

NOTE Depending on the ESP-IDF version, some API names may differ slightly. Use the API names from the HFP AG example included with your own ESP-IDF installation rather than mixing code from different releases. The Bluetooth stack must be enabled before HFP callbacks and initialisation are registered.

# 4. Pair the Bluetooth Headset

Make the ESP32 discoverable:

esp_bt_gap_set_scan_mode( ESP_BT_CONNECTABLE, ESP_BT_GENERAL_DISCOVERABLE );

Put the headset into pairing mode. The ESP32 must then establish, in order:

- Bluetooth pairing

- HFP Service Level Connection

- SCO audio connection

The HFP AG example demonstrates communication with headsets and includes connection and volume-control handling. For early testing, the headset's Bluetooth MAC address can be entered manually:

static esp_bd_addr_t headset_address = { 0x12, 0x34, 0x56, 0x78, 0x9A, 0xBC };

Replace this with the actual Bluetooth address of the headset being used.

# 5. Register Incoming and Outgoing Audio Callbacks

HFP voice audio travels over SCO, not A2DP. One callback is needed for audio received from the headset microphone, and a separate callback supplies audio back to the headset speaker.

Incoming (microphone) audio

static void headset_audio_received( const uint8_t *audio_data, uint32_t length) { // Audio captured from headset microphone // Store samples in a ring buffer. // Pass samples to speech recognition. }

Outgoing (speaker) audio

static uint32_t headset_audio_requested( uint8_t *output, uint32_t requested_length) { // Copy prerecorded PCM response into output. return bytes_copied; }

NOTE Exact callback signatures depend on the ESP-IDF version and the selected audio path — copy them from the installed hfp_ag example rather than retyping from memory. The HFP AG API provides application audio callbacks for the Voice-over-HCI path.

# 6. Add the WROOM #1 UART Code

Create a dedicated UART task in the HFP project to talk to WROOM #1:

#define CONTROL_UART UART_NUM_1 #define CONTROL_RX_PIN 16 #define CONTROL_TX_PIN 17 static void initialise_control_uart(void) { const uart_config_t uart_config = { .baud_rate = 115200, .data_bits = UART_DATA_8_BITS, .parity = UART_PARITY_DISABLE, .stop_bits = UART_STOP_BITS_1, .flow_ctrl = UART_HW_FLOWCTRL_DISABLE, .source_clk = UART_SCLK_DEFAULT }; ESP_ERROR_CHECK( uart_driver_install( CONTROL_UART, 2048, 2048, 0, NULL, 0 ) ); ESP_ERROR_CHECK( uart_param_config(CONTROL_UART, &uart_config) ); ESP_ERROR_CHECK( uart_set_pin( CONTROL_UART, CONTROL_TX_PIN, CONTROL_RX_PIN, UART_PIN_NO_CHANGE, UART_PIN_NO_CHANGE ) ); }

Then continuously read messages coming from WROOM #1:

static void control_uart_task(void *parameter) { uint8_t byte; char message[256]; size_t position = 0; while (true) { int received = uart_read_bytes( CONTROL_UART, &byte, 1, pdMS_TO_TICKS(20) ); if (received <= 0) { continue; } if (byte == '\n') { message[position] = '\0'; process_navigation_message(message); position = 0; } else if (byte != '\r' && position < sizeof(message) - 1) { message[position++] = (char)byte; } } }

Start the task from app_main():

initialise_control_uart(); xTaskCreate( control_uart_task, "control_uart", 4096, NULL, 5, NULL );

# 7. Convert Alerts into Prerecorded Speech

Do not attempt full text-to-speech initially. Instead, store short PCM clips such as:

warning.raw person.raw vehicle.raw obstacle.raw left.raw right.raw ahead.raw one_metre.raw two_metres.raw path_clear.raw

When WROOM #1 sends a message such as:

ALERT=PERSON;DIRECTION=AHEAD;DISTANCE=1450;PRIORITY=NORMAL

WROOM #2 queues the matching clips in sequence:

person.raw ahead.raw one_point_five_metres.raw

The outgoing HFP callback reads these PCM samples from a ring buffer and provides them to the SCO connection.

NOTE Keep the audio format identical to the format requested by the negotiated HFP codec. HFP commonly uses narrowband CVSD or, when supported and negotiated, wideband mSBC — the application must match whatever format the selected ESP-IDF audio path expects.

# 8. Queue Audio Safely

Use a FreeRTOS stream buffer to move prerecorded audio between the application and the outgoing SCO callback without blocking either side:

static StreamBufferHandle_t voice_stream; static void initialise_voice_stream(void) { voice_stream = xStreamBufferCreate(32000, 1); }

Queue a prerecorded response:

static void queue_voice_audio( const uint8_t *data, size_t length) { xStreamBufferSend( voice_stream, data, length, portMAX_DELAY ); }

Outgoing Bluetooth callback — pad with silence if the buffer runs dry mid-request:

static uint32_t provide_headset_audio( uint8_t *output, uint32_t requested_length) { size_t copied = xStreamBufferReceive( voice_stream, output, requested_length, 0 ); if (copied < requested_length) { memset( output + copied, 0, requested_length - copied ); } return requested_length; }

Connect this function using the exact callback-registration method found in the installed HFP AG example.

# 9. Handle the Microphone Command

HFP only delivers microphone audio — it does not automatically understand the sentence spoken into it. There are two development paths from here.

## Easier first version

Use the headset's call/action button, or a physical button, to trigger a scan directly:

uart_write_bytes( CONTROL_UART, "IS_THERE_AN_OBSTACLE\n", strlen("IS_THERE_AN_OBSTACLE\n") );

This lets Bluetooth audio playback be completed and verified before speech recognition is added at all.

## Advanced version

Feed the received microphone PCM into a command-recognition system, limited to a small, fixed set of commands:

- Is there an obstacle?

- What is ahead?

- Check left.

- Check right.

- How far is it?

NOTE Speech recognition is a separate subsystem and may be difficult to run simultaneously with Bluetooth HFP on a single WROOM-32, due to memory and CPU pressure. A practical alternative is to transfer the microphone samples to an ESP32-S3, or use an external voice-recognition module.

# 10. Recommended Implementation Order

Build and test incrementally, in this order — do not combine all sections at once:

1. Run the official HFP AG example

↓

2. Pair the headset

↓

3. Confirm the SCO connection

↓

4. Hear the example test sound

↓

5. Replace the test sound with one prerecorded PCM alert

↓

6. Add UART reception from WROOM #1

↓

7. Play different alerts based on UART messages

↓

8. Add a scan button or headset-button command

↓

9. Add microphone recording

↓

10. Add fixed-command speech recognition

KEY MILESTONE Milestone: pair the headset and successfully play one prerecorded warning through HFP. Everything after that builds on a working audio path.

# Migrating from the Previous Arduino Code

The previous Arduino WROOM #2 code cannot simply have one HFP library line added to it. Its message-parsing logic can be reused, but it must be moved into the official ESP-IDF HFP AG project as a UART task plus an audio-response queue, following the structure laid out in this guide.

Sources: Espressif ESP-IDF HFP AG documentation and API reference; ESP-IDF Bluetooth Classic reference; Espressif hfp_ag example (GitHub).