AI-DRIVEN SHOES FOR THE VISUALLY IMPAIRED

Three-Board System Architecture

ESP32-S3 · WROOM-32 #1 · WROOM-32 #2 — Build Order & Wiring Guide

Three independent programs, one common ground, one spoken interface

# Project Structure

Use this project as three separate programs running on three separate boards. Do not combine all three codes into one file — each board has its own responsibility, its own toolchain, and is developed and tested independently before the next one is connected.

Camera

↓

ESP32-S3 — AI object detection

↓

UART → WROOM-32 #1 — Distance sensing + decision-making

↓

UART → WROOM-32 #2 — HFP Bluetooth audio

↓

↕ Bluetooth headset microphone and speaker

# 1. ESP32-S3 Program

Upload the S3 camera and AI code only to the ESP32-S3. Its job is to:

- Capture images.

- Run your Edge Impulse model.

- Identify the object.

- Determine left, ahead or right.

- Send the result to WROOM #1.

Example message:

OBJECT=PERSON;DIRECTION=AHEAD;CONFIDENCE=0.91

Before uploading, you must add:

- Correct camera pin mapping.

- Edge Impulse model header.

- Two free UART pins.

- Correct model resolution.

NOTE Test it first using the Serial Monitor. Do not connect the other boards until the S3 correctly prints detections.

# 2. WROOM-32 #1 Program

Upload the central-controller code to WROOM #1 using Arduino IDE. Its job is to:

- Receive the AI result from the S3.

- Read the left, centre and right VL53L1X sensors.

- Match the AI direction with the correct distance sensor.

- Decide whether the situation is normal or dangerous.

- Send the final alert to WROOM #2.

Example message:

ALERT=PERSON;DIRECTION=AHEAD;DISTANCE=1450;CONFIDENCE=0.91;PRIORITY=NORMAL

Test the three distance sensors before connecting the S3. The Serial Monitor should display values such as:

Left: 2100 mm Centre: 850 mm Right: 1700 mm

Then connect the S3 and confirm that WROOM #1 receives:

OBJECT=PERSON;DIRECTION=AHEAD;CONFIDENCE=0.91

## S3-to-WROOM #1 connection

S3 TX → WROOM #1 RX S3 RX ← WROOM #1 TX S3 GND → WROOM #1 GND

NOTE TX and RX must be crossed.

# 3. WROOM-32 #2 Program

IMPORTANT The Arduino WROOM #2 code previously given is only for testing UART messages and sentence generation. It does not yet transmit voice through the Bluetooth headset.

For the final version, WROOM #2 should be programmed using ESP-IDF, starting from Espressif's official hfp_ag example. The ESP32 acts as the HFP Audio Gateway, while the Bluetooth headset acts as the hands-free device.

Therefore, the HFP guide does not become a fourth program. It replaces and upgrades the WROOM #2 Arduino program:

Old WROOM #2 Arduino code

↓

Reuse message parsing and sentence logic

↓

Insert into ESP-IDF hfp_ag project

↓

Final WROOM #2 firmware

## What you reuse from the WROOM #2 Arduino code

Reuse these logical parts:

- UART communication with WROOM #1.

- getMessageField()

- objectToSpeech()

- directionToSpeech()

- Alert-message interpretation.

- Scan commands such as IS_THERE_AN_OBSTACLE

You should not directly copy Arduino functions such as Serial.println(), HardwareSerial, String, setup(), and loop() into ESP-IDF without adaptation. In ESP-IDF, they become:

- UART driver functions.

- FreeRTOS tasks.

- Character buffers.

- app_main() instead of setup() and loop().

ESP-IDF lets you assign UART TX and RX pins using uart_set_pin().

# Correct Development Order

Work through these stages in sequence — do not connect the next board until the current stage is confirmed working.

## Stage 1: Test the S3 alone

Upload the S3 program. Use the camera to detect one trained object. Expected output:

Detected: PERSON Direction: AHEAD Confidence: 0.91

NOTE Do not continue until this works.

## Stage 2: Test WROOM #1 sensors alone

Disconnect the S3 and WROOM #2. Upload the central-controller code. Confirm that all three distance sensors give sensible readings by moving your hand in front of each sensor separately.

## Stage 3: Connect S3 to WROOM #1

Connect their UART and common ground. Expected sequence:

S3: OBJECT=PERSON;DIRECTION=AHEAD;CONFIDENCE=0.91 WROOM #1: ALERT=PERSON;DIRECTION=AHEAD;DISTANCE=1450;CONFIDENCE=0.91;PRIORITY=NORMAL

## Stage 4: Test WROOM #2 without Bluetooth

Upload the temporary Arduino WROOM #2 code. Connect it to WROOM #1 through UART. Confirm that it prints:

VOICE OUTPUT: A person is ahead, 1.5 metres away.

At this point, all three controllers and the communication system are working.

## Stage 5: Run the HFP example separately

Create a fresh ESP-IDF project using:

examples/bluetooth/bluedroid/classic_bt/hfp_ag

Set the target and build/flash the original example without changing it:

idf.py set-target esp32

Your first HFP objective is only:

- Pair the Bluetooth headset.

- Establish the HFP connection.

- Establish SCO audio.

- Hear the example audio through the headset.

NOTE The official example is specifically intended to connect an ESP32 acting as an Audio Gateway to an HFP headset or other hands-free client. Do not add your navigation code until the original HFP example works.

## Stage 6: Add WROOM #1 UART reception to HFP

Inside the HFP project, create a FreeRTOS UART task:

WROOM #1 message

↓

UART task

↓

Message parser

↓

Select voice clips

↓

HFP audio queue

↓

Headset speaker

For example, when UART receives:

ALERT=PERSON;DIRECTION=AHEAD;DISTANCE=1450

WROOM #2 queues prerecorded audio clips:

person.raw ahead.raw one_point_five_metres.raw

The HFP audio callback sends those audio samples to the headset.

## Stage 7: Add user input

Initially, use a button or the Bluetooth headset action button. When pressed, WROOM #2 sends:

IS_THERE_AN_OBSTACLE

to WROOM #1. WROOM #1 requests a new scan from the S3 and returns the answer. This gives a complete working interaction:

User presses button

↓

WROOM #2 sends scan command

↓

WROOM #1 reads sensors and requests S3 scan

↓

S3 detects object

↓

WROOM #1 prepares final alert

↓

WROOM #2 speaks through headset

## Stage 8: Add microphone speech recognition last

HFP only transfers the headset microphone audio — it does not understand speech automatically. You still need a speech-recognition system to recognise:

- “Is there an obstacle?”

- “What is ahead?”

- “Check my left.”

- “Check my right.”

NOTE This should be the last stage because it is the most difficult part. A safer progression is: button command → headset button → recorded microphone audio → fixed-command recognition.

# Software Used for Each Board

| Board | Development Platform | Main Program |
| --- | --- | --- |
| ESP32-S3 | Arduino IDE | Camera and Edge Impulse AI |
| WROOM #1 | Arduino IDE | Distance sensors and central control |
| WROOM #2 | ESP-IDF | UART parsing and Bluetooth HFP audio |

# Final Wiring

ESP32-S3 TX → WROOM #1 RX ESP32-S3 RX ← WROOM #1 TX WROOM #1 TX → WROOM #2 RX WROOM #1 RX ← WROOM #2 TX All board grounds → Common GND

NOTE Do not directly connect two TX pins or two RX pins together.

# The Most Realistic First Finished Version

Build the first working prototype with:

- AI object detection.

- Three distance sensors.

- UART communication.

- Bluetooth headset voice output.

- Push button to ask for a scan.

- Prerecorded voice clips.

KEY MILESTONE Only after that version works should you add spoken questions through the headset microphone.

Sources: Espressif ESP-IDF HFP AG documentation and API reference; ESP-IDF UART peripheral reference; Espressif hfp_ag example (GitHub).